STORMWATER MANAGEMENT

TABLE 1

SOUTH GREENSBURG BOROUGH

INLET CAPACITY REDUCTION FACTORS

ASSUMING PARTIAL CLOGGING

Condition	Inlet Type	Reduction Factor*

Sump	Side Opening	0.80

Sump	Grate	0.50

Sump	Combination	0.65

Continuous grade	Side opening	0.80

Continuous grade	Side opening with deflector	0.75

Continuous grade	Longitudinal bars	0.60

Continuous grade	Transverse bars	0.50

Continuous grade	Combination	0.60

NOTES:

*Percentage of theoretical capacity.

STORMWATER MANAGEMENT

TABLE 2

SOUTH GREENSBURG BOROUGH

SPACING OF INSPECTION AND CLEANOUT

STRUCTURES FOR STORM SEWERS

Size of Pipe	Maximum Allowable Spacing

(Inches)	(Feet)

15	400

18 - 36	500

42 - 60	700

66 or larger	Unlimited